# Blackpool Council Engineering (Trams)

# Rail replacement on Blackpool and Fleetwood Tramway

Revision History

| Version | Date | Description of Change |
| --- | --- | --- |
| Draft | October 2014 | First Draft for comment |
| 1.0 | June 2020 | Initial Issue |

| BC Document Number: | BPL-TWY-GD-002 |
| --- | --- |
| Issue Number: | 1.0 |
| Issue Date: | June 20 |
| Author: | Blackpool Council, Track Services |
| Approver: Signature: | Tramway Safety Officer |
| Review Date: | June 21 |

Record of Review

| Date of Review | Occasion A- Annual X- Exceptional | Name of Reviewer | Signature |
| --- | --- | --- | --- |

Issue Record:

Copy No. 1	Track Safety Officer

Copy No. 2	Blackpool Council Occupational Health & Safety Department

Copy No. 3	Blackpool Transport Services

Contents

1	Introduction	6

1.1	Background	6

1.2	Roles	6

2	Permanent Way	7

2.1	Network Layout	7

2.2	Plain Line Track Construction	8

2.2.1	Track Gauge	8

2.2.2	Minimum curve radii	8

2.2.3	Ballasted Track Construction	8

2.3	Switch and Crossing Track Construction	9

3	Wheel Rail Interface	11

3.1	Compatible Wheel Profile	11

4	Platforms & Structural Clearances	12

4.1	Tram Stops	12

4.2	Structures	12

5	Track Access	14

5.1	Guidance	14

5.2	Stabling & Security	14

6	Requirements Specification	16

6.1	Scope	16

6.1.1	Scope of work/special precautions	17

6.2	Possession Strategy	17

6.2.1	Protection Arrangements	17

6.2.2	Permit Applications	17

6.3	Working Safely on the Tramway	17

6.3.1	Restricted working hours	18

6.4	Suggested Methodology	18

6.5	Tram Stops and crossings	18

6.6	Ballasted track	18

6.7	Technical & Functional Requirements	19

6.7.1	Pre-bent rails	19

6.7.2	Outer worn rail	19

6.7.3	Sleepers	19

6.7.4	Tamping	19

6.8	Road closures	19

6.8.1	Track crossing installations	19

6.9	Contact	20

List of Figures

Figure 1 Blackpool & Fleetwood Tramway Network Layout	7

Figure 2 Ri60R1 Groove Rail Profile	9

Figure 3 Crossing Nose & Check Rail Flange Running Profiles	10

Figure 4 Compatible Wheel Profile	11

Figure 5 Critical Wheel Profile Dimensions	11

Figure 6 Platform Edge Clearances	12

Figure 7 OLE Centre Pole Minimum Clearance	13

Figure 8 Tamper On Tracking	14

Figure 9 Melton Place	15

Figure 10 Orion Curve	15

List of Tables

Abbreviations

BC		Blackpool Council

BTS		Blackpool Transport Services

IM		Infrastructure Manager

OLE		Overhead Line Equipment

ORR		Office of Rail Regulation

ROGS		Railways and Other Guided Transport Systems (Safety) Regulations 2006

RSP2		Guidance on Tramways

RRV		Road Rail Vehicle

TSO		Tramway Safety Officer

TO		Transport Operator

TU		Transport Undertaking

References

RSP2, Guidance on Tramways

URL: http://orr.gov.uk/__data/assets/pdf_file/0018/2637/rspg-2g-trmwys.pdf

Tramway Technical Guidance Note 1 Design Requirements for Street Track

URL: http://orr.gov.uk/__data/assets/pdf_file/0019/5068/ttgn1-StreetTrack.pdf

# Introduction

## Background

The Blackpool & Fleetwood Tramway is the only remaining first generation tramway in the United Kingdom, having remained in constant operation since it first opened in 1885.   The current network extends for 18.2Km along the Fylde coastline from Starr Gate terminus at the southern end of the alignment to Fleetwood Ferry in the north.

Blackpool & Fleetwood Tramway is owned by Blackpool Council (BC) and operated under licence by Blackpool Transport Services (BTS).

## Roles

The Railway and Other Guided Transport Systems (Safety) Regulations 2006 places legal obligations on persons carrying out specific duties under the regulations.  These bodies are called Duty Holders.  The Duty Holder’s for Blackpool & Fleetwood Tramway are:

- Infrastructure Manager - Blackpool Council Track Safety Officer

- Transport Operator – Blackpool Transport Services Head of Tramway

The Infrastructure Manager is responsible for the condition of the asset and is the principal point of contact for all matters relating to the tramway.

# Permanent Way

## Network Layout

The track layout is as shown in Figure 1.

Figure 1 Blackpool & Fleetwood Tramway Network Layout

Ballasted track construction is used on Blackpool & Fleetwood tramway between Uncle Tom’s Cabin and Fisherman’s Walk only.  However, the ballast track is punctuated at regular intervals by short sections of paved track construction types between these tram stops.  Therefore there is a requirement for plant that cannot easily disengage steel wheel contact with the rails to exhibit rail wheel interface characteristics that are compatible with the requirements set out in this section of this Guidance Document, especially through the embedded Switch & Crossing (S&C) Permanent Way configuration.

## Plain Line Track Construction

Track on United Kingdom tramways is classified as integrated and segregated depending upon the environment in which it operates.  Track on the Blackpool & Fleetwood tramway is fundamentally of two construction types:

- Ballasted track form to Network Rail Track Construction Standard to satisfy the requirements for Category 4; and

- Paved track with encapsulated groove rail (Ri60R1).

All Switch & Crossing units are constructed in paved track sections of the alignment and Tamper & Road Rail Vehicle (RRVs) wheel profiles proposed to be used on the network will need to be compatible with both the plain line Ri60R10 and S&C in respect of rail groove and gauge corner compliance.

### Track Gauge

Track gauge is 1435mm throughout.

### Minimum curve radii

The minimum curve radii on the network are as follows for the different types of track construction:

- Ballasted track 147m; and

- Paved Track 22.5m.

### Ballasted Track Construction

The ballast track on the network is constructed from the following materials:

- Rail profile predominantly CEN56E1 with some 95lb bullhead rail on tight curves (see Note);

- Rail inclination 1:20 nominal;

- Sleeper spacing nominally 700mm (26 per 18.288m);

- Sleeper type G44 except for tight radius curves where timber sleepers are used with check rail; and

- Depth of ballast 250mm nominal.

Note: At Thornton Gate and Cumberland Avenue there are extended lengths of vertically mounted Ri60R1 on 001 Bearers.  Note that the gauge corner for the asymmetric groove rail used on the network is 10mm.  The groove is 36mm wide at its maximum width and 47mm deep. The detailed drawing is shown in Figure 2.

Figure 2 Ri60R1 Groove Rail Profile

## Switch and Crossing Track Construction

S&C construction is important because tampers and rail mounted plant that ideally need to stay on the rail head to move from site to site must be compatible with the changes in the rail configuration as the wheel flange passes through the crossing nose and parallel check rail section.  At this point the groove width and depth alter significantly to present a rail profile to the wheel that forces the wheel out of the groove and generates ‘flange running’ during which the flange makes contact and runs on the shallow (circa 14mm) rail channel.  Figure 3 illustrates the changes in the rail groove profile through all S&C crossing noses and check rail sections and the transitional arrangements between full Ri60R1 groove rail.

Figure 3 Crossing Nose & Check Rail Flange Running Profiles

# Wheel Rail Interface

## Compatible Wheel Profile

Figure 4 illustrates a proven compatible wheel profile for plant utilised on the Blackpool & Fleetwood tramway during the 2009-2011 track renewal works.

Figure 4 Compatible Wheel Profile

Figure 5 tabulates the critical dimensions of the proven wheel profile illustrated at Figure 4.

Figure 5 Critical Wheel Profile Dimensions

# Platforms & Structural Clearances

## Tram Stops

Blackpool & Fleetwood Tramway complies with the Mobility Inclusive Regulations and Rail Vehicle Accessibility Regulation through the provision of low level platforms across the network with stepping distances nominally 50mm between tram and stop edge in both the vertical and horizontal planes.  Figure 6 identifies the setting out clearances that are applied at all tramstops across the network.

Figure 6 Platform Edge Clearances

## Structures

The minimum clearance to any structure is to centre mounted Overhead Line Equipment (OLE) poles.  The clearance to these structures is set at Developed Kinematic Envelope (DKE)+100mm  (minimum) based on the DKE of the Flexity 2 tram. This means that for straight and level track the closest point of a centre mounted OLE pole can be 857.5mm from the running edge of the 6ft rail (1575mm from the track centre line).  As dynamic effects of track curvature and can’t influence different vehicles in different ways it is the responsibility of the prospective supplier to ensure that gauging clearances are satisfied for operation on the Blackpool & Fleetwood tramway.  It should however be noted that the structural clearances applied on the network are both consistent with the requirements of RSP2 and those installed on all other UK light rail networks. Figure 7 is an extract from RSP2 that illustrates the minimum clearances.

Figure 7 OLE Centre Pole Minimum Clearance

# Track Access

## Guidance

Standard precautions and procedures for gaining access to the network are contained in the document ‘Safety on the Line’, which is available upon request from the TSO.  Amongst other things it sets out the minimum OLE wire height for the network and the standard safe working heights to be adopted.

Figure 8 shows a tamper on tracking at Fisherman’s Walk close to the northern limit of the ballast track construction.

Figure 8 Tamper On Tracking

The selection of the most appropriate on and off tracking point requires careful consideration due to the close proximity of residential properties and constricted turning opportunities for Long Wheelbase vehicles.  Fisherman’s Walk is a good choice of location because it represents the boundary between integrated on street and segregated off street track construction types.

## Stabling & Security

All areas of the tramway between Uncle Tom’s Cabin and Fisherman’s Walk are entirely accessible with no secure area available for stabling Tampers or other large items of plant.  The most appropriate stabling areas are at Thornton Gate or Bispham.  Access to the secure tram depot at Starr Gate is impractical because of the nature of the network south of Uncle Tom’s Cabin and the tight radius curves within the depot.  The requirements for stabling and security should be discussed with the TSO during the consultation process which will happen post-award and prior to commencement on site.

Figure 9 – Orion Curve

Figure 10 – Melton Place

# Requirements Specification

## Scope

This section outlines the key information necessary to enable interested suppliers to develop a technical proposal to undertake rail replacement on the Blackpool & Fleetwood tramway system. Applications to Blackpool Council should include an indicative programme of works, a schedule of rates and an offer price.

### Scope of work/special precautions

The work areas shall be surveyed prior to ordering new rail. Any crossings where underrun needs to be removed shall be identified prior to commencement. Additionally, the pedestrian crossings formed in hard materials will also need to be identified and removed by the Council to necessitate the work.

All platforms shall remain untouched. Please note restricted access when passing through the platforms.

Other works are programmed during the system closure, this includes sub arc welding and tamping. Refer to the other appropriate lots to clarify where the proposed works are and what if any impact it will have on the work.

## Possession Strategy

Works on the tramway are to be undertaken outside the resorts tourist and illumination season. The preferred date for undertaking the works is during the February half term holiday (week commencing  15th Feb 2021).  A detailed works programme is required to enable the Tramway Safety Officer to establish whether the required access can be negotiated with the Transport Operator.

Contractors must be aware of other potential works during this period including tamping (Uncle Tom – Fishermans Walk) and sub-arc welding which may be undertaken by other Contractors.  The successful Contractor should liaise with BC to avoid scheduling conflicts.

### Protection Arrangements

Rail replacement will be under full possession (system closed).  Unless specifically arranged in advance the OHLE is LIVE AT ALL TIMES. Full details of the definitions and requirements for establishing safe systems of work are contained in Blackpool Council’s track safety publication, ‘Safety on the Line’.  This document is the equivalent of the Rule Book GE/RT8000.

### Permit Applications

Permits should be submitted to the Tramway Safety Officer.  A copy of the Permit Application Form, including information on how to submit can be found via the following URL:

https://www.blackpool.gov.uk/Business/Licensing-and-permits/Documents/Tramway-Activity-Permit-Application.pdf

## Working Safely on the Tramway

Full details of how to implement safe systems of work on the tramway including working adjacent and under overhead line electrification equipment is contained within the document ‘Safety on the Line’.  A condensed version of the document outlining the salient points is available on Blackpool Council’s website via the following URL:

https://www.blackpool.gov.uk/Business/Licensing-and-permits/Documents/tramway-overhead-powerlines.pdf

### Restricted working hours

Some work may be conducted at night to allow a tram service during the normal working day. Trams cease to operate between 01:00 to 05:30, however the OHLE remains LIVE. Advanced notice must be given for any out of hours work proposed.

## Suggested Methodology

Where possible, a Highways lane possession will be acquired to assist in loading/storage and rail installation operations. During the work, if safe to do so and access allows, the other line may be used to operate the service where the trams will run bang line. Where access to the site is restricted, the operational line will be closed for work plant access (road/rail machines)

The network layout map indicates where the turnback locations are but for clarity these are scheduled out here:

- Bispham;

- Little Bispham;

- Cleveleys; and

- Thornton Gate.

Once work is complete beyond the recognised turnback point the line block will be shortened to enable revenue service to be extended.

## Tram Stops and crossings

Melton Place site will require the removal of a pedestrian crossing. Upon completion, the crossing and underrun protection (supplied by the Council) will be reinstated.

Any road rail plant required to undertake the work must ensure adequate clearance to the platform edge

## Ballasted track

The areas of relay are;

Flat bottom 113lb (56E1)

Location		pole nr			length	    	track	sleeper

Melton Place     		LT10.231 – 354   	123m	     	East	G44

Melton Place		LT10.259 – 354   	95m          	West	G44

Cleveleys Crescent	LT11.362 – 435		73m		East	G44

Maplewood Drive	LT10.555 – 782		227m		East	G44

Bullhead 95lb rail

Orion Curve		LT11.207 – 307		100m		west	timber

(note check rail present in this location)

The majority of the existing infrastructure (sleepers/ballast etc) does not require replacing. Although a small number of G44 sleepers with damaged fastclip housings will require replacing (10 maximum). The sleepers will be provided by the Council. The scope of the work is to replace the outer rail of each bend. At the start and end of the rail replacement there is an existing plated joint.

## Technical & Functional Requirements

### Pre-bent rails

The contractor, in adequate time, must survey the area and determine the radius of each bend and order the pre-bent rail accordingly.

The system is continuously welded rail (CWR) – There will be 1 fish plated joint followed by 2 welded joints.

### Outer worn rail

Only the outer rail (in relation to direction of travel) will be replaced.

### Sleepers

It is envisaged that there are a maximum of 10 G44 concrete sleepers that require replacing. This shall be undertaken by the contractor following the installation of the new rail. Sleepers will be provided by the Council. All the timber sleepers and chairs in the bullhead section are in good condition.

### Tamping

Following installation of the new rail, the area of replaced rail +50m in each direction will require tamping.

## Road closures

Road closures will not be required, however lane possessions may be required and if the contractor requires them to facilitate the work. Any lane possessions must be requested 8 weeks in advance so the Council can arrange the appropriate permits/traffic management.

### Track crossing installations

It is not envisaged that any works will be required in the vicinity of underrun protection or vehicle crossings. For information, proprietary rubber mat crossings are installed at the following locations:

- Road (vehicular):

  - Bispham

  - Norbreck

  - Anchorsholme Lane

  - Westmoreland Avenue

  - College Farm

  - Rossall Lane

  - South Strand

  - Larkholme Avenue

- Pedestrian:

  - Vicarage Ave

  - Clarence Ave

  - Stockdove Way

- Platforms – in ballasted areas only.

## Contact

All enquiries to undertake works on the tramway should be directed to the Tramway Safety Officer at Blackpool Council’s Layton Depot site at the following address:

Plymouth Road

Blackpool

FY3 7HW

Full contact details for Blackpool Council can be found via the following URL:

https://www.blackpool.gov.uk/Contact-us.aspx.