| Breast High Road Borrowdale Restoration Project - Images |  |  |  |
| --- | --- | --- | --- |
| Map Reference Point | Current feature/condition | Proposed Access Improvement | Image |
| 42-43 | 9 inch culvert pipe no longer working | Replace 9inch culvert - 4 lengths, 3 joints |  |
| 44 | Side drain into culvert, currently eroded | Replace drain and ensure protected edge. 10m |  |
| 45 | Bend in track unsustainable with loose rocks on steep surface | Create an aggregate surface around bend. |  |
| 46 | 9 inch culvert pipe broken | Replace 9 inch culvert pipe at a gentler angle |  |
| 47 | 9 inch culvert pipe broken | Replace 9 inch culvert pipe |  |
| 48-49 | Drain | Clear out drain leaving track so functions properly. 8m |  |
| 50-51 | Side drains on both sides to clear | Clear out drain so functions correctly and use material to raise road surface. 105m |  |
| 52-53 | Drainage ditch and borrow pit for materials for surfacing |  |  |
| 54 | No drainage on track | Install cross drain |  |
| 55-56 | Side drain | Clear out drain so functions correctly. Use material on track 30m |  |
| 57 | no drainage on track | Install cross drain |  |
| 58 | no drainage on track | Install cross drain |  |
| 59 | Reinstate side drain that is blocked to corner | Clear out drain so functions correctly. Use material on track 35m |  |
| 60 | Cross drain not working | Remove cross drain |  |
| 61 & 62 | Existing cross drain not working | Remove and replace with two cross drains. |  |
| 63 | 12 inch culvert working | Culvert ok - leave as is |  |
| 64-66 | Start of Deep erosion of track and loss of material | Earthworks - fill and reprofile track 40mx12m |  |
| 65 | Deep erosion of ditch to side of route | Earthworks - reprofile ditch 20m |  |
| 67 | Cross drain required | Install cross drain |  |
| 68-69 | Side drain not functioning | Install side drain along northern side. Use material on track to raise camber. 200m |  |
| 70 | 12 inch culvert ok | Culvert ok - leave as is |  |
| 71 | Cross drain not working | Remove cross drain |  |
| 72-73 | Existing culverts blocked and water eroding track. | Remove 4m wall and create 3mx3m sediment trap 1.5m deep. Flag bottom and wall up. 20mx3m pitched area including 2 fords. Revetment 2m on lower side. Est. 60t stone required |  |
| 74-75 | Side drain blocked | Reinstate side drain. Use material on track 450m. |  |
| 76 | Bank eroded | Earthworks - realign ditch and build embankment to support track 20mx4m |  |

Pitching Stone:

It is estimated ‘Borrowdale’ will require 63 tonnes of stone for the pitching, and culvert headwalls.

Subsoiling:

We will also require subsoiling and surfacing of 500 metres (x 3 metre width) of track within the ‘Borrowdale’ section. See below for map.

We have identified potential sources of substrate which could also be used during the subsoiling process – subject to agreement with the site supervisor and landowner.

The subsoiling will also require a 6inch (15cm) capping of aggregate material in addition to the subsoiling (Est. Total for ‘Borrowdale’ of 225 tonnes of aggregate)